NOTICE

DISCLAIMER. The information contained in this publication is subject to constant review in the light of changing government requirements and regulations. No subscriber or other reader should act on the basis of any such information without referring to applicable laws and regulations and/or without taking appropriate professional advice. Although every effort has been made to ensure accuracy, the International Air Transport Association shall not be held responsible for any loss or damage caused by errors, omissions, misprints or misinterpretation of the contents hereof. Furthermore, the International Air Transport Association expressly disclaims any and all liability to any person or entity, whether a purchaser of this publication or not, in respect of anything done or omitted, and the consequences of anything done or omitted, by any such person or entity in reliance on the contents of this publication.

© International Air Transport Association. All Rights Reserved. No part of this publication may be reproduced, recast, reformatted or transmitted in any form by any means, electronic or mechanical, including photocopying, recording or any information storage and retrieval system, without the prior written permission from:

Senior Vice President

Operations, Safety and Security

International Air Transport Association

800 Place Victoria

P.O. Box 113

Montreal, Quebec

CANADA H4Z 1M1

ISSA Checklist
ISSM Part I Edition 7

© 2022 International Air Transport Association.All rights reserved.

Montreal—Geneva

# Section 3 —  Operational Control and Flight Dispatch (DSP)

Applicability Section 3 addresses the requirements for operational control of flights conducted by single and multi-engine aircraft and is applicable to an operator that conducts such flights, whether operational control functions are conducted by the operator or conducted for the operator by an external organization (outsourced). Specific provisions of this section are applicable to an operator based on the operational system in use, the manner in which authority is delegated by the operator, and the responsibilities, functions, duties or tasks assigned to the personnel involved. The ISSA standards and recommended practices (ISARPs) in Section 3 are applicable only to those aircraft that are of the type authorized in the Air Operator Certificate (AOC) or equivalent document, and are utilized in commercial and non-commercial passenger and/or cargo operations Certain ISARPs are also applicable to ferry, test and training flights even though they refer to a non-revenue-generating flights, and such application is indicated in a note that is part of the standard or recommended practice. Table 3.1 categorizes the personnel that are delegated the authority to exercise operational control, assigned the overall responsibility for the overall operational control of a flight, assigned the individual responsibility to carry out one or more functions, duties or tasks related to the operational control of a flight, or assigned the duty to provide administrative support to others with responsibilities related to operational control. Table 3.5 defines the competencies of operational control personnel appropriate to the assignment of overall responsibility for operational control and/or to carry out one or more operational control functions, duties or tasks according to their specific competencies. All personnel utilized to perform operational control functions as defined in Table 3.1, or that act in a manner consistent with the functional categories specified in Table 3.1 and the competencies specified in Table 3.5, irrespective of management or post holder title, are subject to specified training and qualification provisions in this section relevant to the operational control function performed. Individual DSP provisions, and/or individual sub-specifications within a DSP provision, that: - Do not begin with a conditional phrase are applicable to all operators unless determined otherwise by the Auditor. - Begin with a conditional phrase (“If the Operator...”) are applicable if the operator meets the condition(s) stated in the phrase. The conditional phrase serves to define or limit the applicability of the provision (e.g. “If the operator utilizes…” or “If an FOO or FOA is utilized…”). - Begin with a conditional phrase that specifies the use of a Flight Operations Officer (FOO) by an operator are applicable when the operator assigns the FOO, as defined in the IRM and delegated authority in accordance with Table 3.1, responsibility to carry out operational control functions, duties or tasks related to all of the competencies of operational control as specified in Table 3.5. - Begin with a conditional phrase that specifies the use of a Flight Operations Assistant (FOA) by an operator are applicable when the operator assigns the FOA, as defined in the IRM, responsibility to carry out operational control functions, duties or tasks related to one or more, but not all, competencies of operational control as specified in Table 3.5. - Are applicable to all systems of operational control, but with differences in application to each system, will have those differences explained in the associated Guidance Material (GM). - Contain the phrase “personnel responsible for operational control” or “personnel with responsibility for operational control” refer to any suitably qualified personnel with responsibility for operational control as designated by the operator, to include the pilot-in-command (PIC) unless otherwise annotated. - Contain training and qualification requirements are applicable to personnel, other than the PIC, that are assigned responsibilities related to the operational control of flights. PIC training and qualification requirements for all systems of operational control are specified in ISM Section 2 (FLT). - Are eligible for conformance using variations, including Operational Variations approved by the Authority, that contain a note referring to the additional SRM and safety monitoring requirements necessary to ensure an acceptable level of safety is maintained Where operational control functions, duties or tasks are outsourced to external service providers, an operator retains overall responsibility for operational control and will have processes to monitor applicable external service providers in accordance with ORG 2.2.1 located in Section 1 of this manual to ensure requirements that affect operational control are being fulfilled.

General Guidance Authority and Responsibility For the purposes of this section authority is defined as the delegated power or right to command or direct, to make specific decisions, to grant permission and/or provide approval, or to control or modify a process. For the purposes of this section responsibility is defined as an obligation to perform an assigned function, duty, task or action. An assignment of responsibility typically also requires the delegation of an appropriate level of authority. Operational Control Operational control is defined as the exercise of authority to initiate, continue, divert or terminate a flight in the interest of the safety and security of the aircraft and its occupants. An operator may delegate the authority for operational control of a specific flight to qualified individuals, but typically retains overall authority to operate and control the entire operation. An operator may also assign the responsibility to carry out specific operational control functions, duties, or tasks related to the conduct of each flight to identifiable, qualified and knowledgeable individual(s), but always remains responsible (and accountable) for the conduct of the entire operation. Any individuals delegated the authority to make specific decisions regarding operational control would also be responsible (and accountable) for those decisions. Additionally, individuals assigned the responsibility to carry out specific operational control functions, duties, or tasks related to the conduct of each flight are also responsible (and accountable) for the proper execution of those functions, duties, or tasks. In all cases, the authority and responsibility attributes of operational control personnel are to be clearly defined and documented by the operator, and communicated throughout the organization. It is important to note that when an operator assigns the responsibility for functions, duties or tasks related to the initiation, continuation, diversion and termination of a flight to employees or external service providers, such operator retains full responsibility (and accountability) for the proper execution of those functions, duties or tasks by ensuring: - The training and qualification of such personnel meets any regulatory and operator requirements; - Personnel are performing their duties diligently; - The provisions of the Operations Manual are being complied with; - An effective means of oversight is maintained to monitor the actions of such personnel for the purpose of ensuring operator guidance and policy, and regulatory requirements, are complied with. Authority for the Operational Control of Each Flight In order to practically exercise operational control of flight operations, an operator typically delegates the authority for the initiation, continuation, diversion or termination of each flight to qualified individuals. Such delegation occurs in conjunction with an operator's overall system of operational control as follows: - Shared systems, wherein operational control authority is shared between the pilot-in-command (PIC) and a flight operations officer/flight dispatcher (FOO) or designated member of management, such as the Director of Flight Operations (or other designated post holder); For example: The FOO (or designated member of management, as applicable) has the authority to divert, delay or terminate a flight if in the judgment of the FOO, a designated member of management or the PIC, the flight cannot operate or continue to operate safely as planned or released. - Non-shared systems, wherein operational control authority is delegated only to the PIC. For example: Only the PIC has the authority to terminate, delay, or divert a flight if in the judgment of the PIC the flight cannot operate or continue to operate safely as planned. Responsibility for Operational Control of Each Flight While an operator always retains full responsibility (and accountability) for the entire operation, the responsibility for the practical operational control of each flight is typically assigned to qualified individuals. As with the delegation of authority, the assignment of responsibility related to the operational control of each flight occurs in conjunction with a system of operational control as follows: - Shared systems, wherein operational control responsibility for each flight is shared between the PIC and an FOO, or between the PIC and a designated member of management such as the Director of Flight Operations (or other designated post holder). In either shared system, the PIC, FOO or designated member of management, as applicable, may be assisted by other qualified personnel assigned the individual responsibility (by the operator) to carry out specific operational control functions, duties or tasks. Such personnel, however, typically do not share operational control responsibility with the PIC, FOO or designated member of management, as applicable. For example: The FOO (or designated member of management) and the PIC are jointly responsible (and accountable) for the functions, duties or tasks associated with the operational control of a flight, such as pre-flight planning, load planning, weight and balance, delay, dispatch release, diversion, termination, etc. In such systems the FOO (or designated member of management) may carry out such responsibilities unassisted or be assisted by qualified personnel assigned the individual responsibility (by the operator) to carry out specific operational control functions, duties or tasks. - Non-shared systems, wherein the PIC is solely responsible for all duties, functions, or tasks regarding operational control of each flight, and may carry out such responsibilities unassisted or be assisted by qualified personnel assigned the individual responsibility (by the operator) to carry out specific operational control functions, duties or tasks. For example: The PIC is solely responsible (and accountable) for the duties, functions, duties or tasks associated with the operational control of a flight, and the PIC either acts unassisted or is assisted by qualified personnel in carrying out functions, duties or tasks such as preflight planning, load planning, weight and balance, delay, dispatch release, diversion, termination, etc. Responsibility for Individual Operational Control Functions, Duties, or Tasks It is important to note that, except for purely non-shared (PIC only) system, and as illustrated by the examples in the previous paragraph, the assignment of responsibilities related to the operational control of each flight can be further subdivided among a number of qualified and specialized personnel. In such cases, the responsibility for individual or specific operational control functions, duties or tasks is typically assigned to FOA personnel who support, brief and/or assist the PIC, FOO personnel and/or designated member(s) of management, as applicable, in the safe conduct of each flight. Examples of such qualified personnel include Weather Analysts, Navigation Analysts/Flight Planning Specialists, Load Agents/Planners, Operations Coordinators/Planners, Maintenance controllers and Air Traffic Specialists. Note: . Some operators might choose to assign the responsibility for specialized operational control functions, such as those described in the example, to fully qualified FOO personnel. In such cases, an FOO, although qualified in all competencies of operational control, would be functionally acting as an FOA. Therefore, for the purpose of an audit, FOO personnel acting in this limited capacity are assessed as FOA personnel. Note: . Load Agents/Planners/Controllers who perform load control functions within the scope of ground handling operations may not be considered FOAs if trained and qualified in accordance with ISM Section 6 (GRH), Subsection 2.1, Training Program. Administrative Support Personnel FOA personnel are not to be confused with administrative personnel that lack any operational control authority, have very limited operational control responsibilities, and who simply provide, collect or assemble operational documents or data on behalf of the PIC, the FOO, designated member of management or the operator. Administrative personnel may be present in any system of operational control, are excluded from the initial and continuing qualification provisions of this section, and may be qualified as competent through on-the-job training (OJT), meeting criteria as specified in a job description, or through the mandatory use of written instruments such as task cards, guidelines, or checklists. Additional Note For the purposes of this section, continuing qualification includes recurrent or refresher training as well as any training necessary to meet recency-of-experience requirements. Definitions, Abbreviations, Acronyms Definitions of technical terms used in this ISSM Section 3, as well as the meaning of abbreviations and acronyms, are found in the IATA Reference Manual for Audit Programs (IRM).

## 1 Management and Control

### 1.3 Authorities and Responsibilities

DSP 1.3.4

The Operator shall delegate the authority for operational control of each flight only to the PIC in a non-shared system of operational control, or to a combination of suitably qualified individuals in a shared system of operational control, to include the PIC and either: - An FOO in a shared system of operational control that requires the use of FOO personnel, or - A designated member of management or Post Holder in a shared system of operational control that requires the use such management personnel. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified specific system for operational control of flights as required by regulations. ☐ Identified/Assessed operational control system (focus: specific type of shared/non-shared operational control system in accordance with regulatory requirements). ☐ Interviewed responsible operational control manager(s). ☐ Examined job description for positions with delegated authority for operational control of flights (focus: authority/responsibilities appropriate for specific type of shared/non-shared system of operational control. ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Flight Monitoring. Refer to General Guidance in the beginning this section for the definition of Authority in the context of operational control. The intent of this provision is to ensure an operator delegates the authority to initiate, continue, divert or terminate a flight in the interest of the safety and security of the aircraft and its occupants (operational control) only to appropriately qualified individuals. Examples of operational control systems are provided in the following table as a means to identify how authority is typically delegated by an operator. System of Operational Control; Location; System Description. Shared System (General); (i), (ii); Operational control authority is shared between the PIC and a flight operations officer/flight dispatcher (FOO) or a designated member of management.. Full Shared System (PIC and FOO); (i); The PIC and FOO have joint authority over the decisions functions, duties or tasks associated with the operational control of a flight. Such systems are characterized by the use of flight monitoring and a dedicated communications system (voice or electronic) separate from the ATC system in order to maintain shared authority.. Partial Shared System (PIC and FOO); (i); The PIC and FOO have joint authority over all preflight decisions, functions, duties or tasks associated with the operational control of a flight, but during flight the PIC has sole authority. Such systems typically include an agreed point of transition from joint to sole responsibility (e.g. pushback or throttle advance for takeoff). This point of transition also typically coincides with the point when the MEL is no longer applicable and flight crews transition to inflight procedures. Partial Shared systems are characterized by the use of flight monitoring if required by the Authority or desired by the Operator and typically lack the dedicated communications system necessary to maintain shared authority in flight.. Shared System (PIC and Management); (ii); Functionally equivalent to a full-shared system except that the PIC and a designated member of management, often the Director of Flight Operations or any suitably qualified and knowledgeable member of management designated by the operator, have joint authority over the decisions, functions, duties or tasks associated with the operational control of a flight.. Non-shared System (General); Main standard; Operational control authority is delegated only to the PIC who may or may not be assisted by other support personnel.. Non-shared System (PIC-only); Main standard; The PIC has sole authority over any and all decisions and completes all tasks (unassisted) related to the operational control of each flight. This does not preclude administrative personnel from providing, collecting or assembling operational documents or data related to each flight on behalf of the PIC and as defined in Table 3.1. Such systems may employ flight monitoring if required by the Authority or desired by the operator.. Non-shared System (PIC-assisted); Main standard; The PIC has sole authority over any and all decisions regarding operational control. However, the PIC is assisted by others (e.g. FOO, FOA or a member of management) that lack operational control authority, but are assigned the responsibility to carry out specific functions, duties or tasks, such as flight planning, flight support, briefing and in-flight monitoring. Such systems employ flight monitoring if required by the Authority or desired by the operator.. Note : An FOA can be utilized in combination with FOOs or designated members of management in all systems of operational control. If such personnel are delegated authority in a shared system, however, it would be limited to their specific area of competency.. Table 3.1 categorizes operational control personnel, defines their authority, identifies their responsibilities and illustrates the relationship of such responsibilities to the operation as a whole.

DSP 1.3.5

The Operator shall retain the overall responsibility for operational control of each flight and assign the responsibility to carry out functions, duties or tasks related to the operational control of each flight only to the PIC, or to a combination of suitably qualified personnel as defined in Table 3.1, to include the PIC and as applicable to the system of operational control responsibility: - If the Operator has a shared system of operational control responsibility, either of the following: - An FOO, who shares overall operational control responsibility with the PIC and/or supports, briefs and/or assists the PIC in the safe conduct of each flight, or - A designated member of management or Post Holder who shares overall operational control responsibility with the PIC and/or supports, briefs and/or assists the PIC or FOO in the safe conduct of each flight. Note: FOA and/or administrative personnel can be utilized in combination with FOOs and/or designated members of management in a shared system of operational control, but neither would share operational control responsibility with the PIC, FOO or designated member of management. - If the Operator has a non-shared system of operational control responsibility, one or more of the following: - An FOO who supports, briefs and/or assists the PIC in the safe conduct of each flight, or - A designated member of management or Post Holder who supports, briefs and/or assists the PIC or FOO in the safe conduct of each flight, or - FOA personnel who support, brief and/or assist the PIC or FOO in the safe conduct of each flight, and/or - Administrative personnel who do not support, brief and/or assist the PIC or FOO, but provide, collect or assemble operational documents or data relevant to the conduct of each flight. (GM) Note: An operator may choose to assign limited responsibilities to fully qualified FOO personnel, or to utilize them only to carry out individual or specific operational control functions, duties or tasks. In such cases, an FOO would be functionally acting as an FOA.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed operational control system (focus: operator has overall responsibility for operational control; responsibilities for individual functions/duties/tasks assigned to positions as specified in Table 3.1. ☐ Interviewed responsible operational control manager(s). ☐ Examined job description for positions with responsibility for individual operational control functions/duties/tasks (focus: position responsibilities appropriate for specific type of shared/non-shared system of operational control). ☐ Observed operational control/flight dispatch operations (focus: responsibilities for individual functions/duties/tasks). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to General Guidance in the beginning this section for the definition of Responsibility in the context of operational control. The intent of this provision is to specify the various ways operational control responsibilities can be assigned by an operator and to ensure only suitably trained and qualified individuals, in addition to the PIC, are assigned overall responsibility for operational control or the responsibility to carry out one or more functions, duties or tasks related to the operational control of each flight. The specifications of this provision apply irrespective of post holder titles or whether personnel positions are described in the OM. If personnel are assigned the responsibility to carry out operational control functions, duties or tasks, and act in a manner consistent with the specifications of this provision or the descriptions found in Table 3.1, the specifications of this provision are applicable, as well as the specifications of ensuing provisions that require such personnel to be trained and qualified for the operational control responsibilities, functions, duties or tasks that they are performing. Examples of operational control systems are provided in the following table as a means to identify how responsibility is typically assigned by an operator. System of Operational Control; Item; System Description. Shared Systems (General); (i) (a), (i) (b); Operational control responsibility is shared between the PIC and an FOO or designated member of management.. Full Shared System (PIC and FOO); (i) (a); The PIC and FOO are jointly responsible for the decisions, functions, duties or tasks associated with the operational control of a flight. Such systems are characterized by flight monitoring and a dedicated communications system (voice or electronic) separate from the ATC system in order to maintain joint responsibility.. Partial Shared System (PIC and FOO); (i) (a); The PIC and FOO are jointly responsible for all preflight decisions, functions, duties or tasks associated with the operational control of a flight, but during flight the PIC has sole responsibility. Such systems are characterized by the use of flight monitoring if required by the Authority or desired by the Operator and typically lack the dedicated communications system necessary to maintain shared responsibility in flight.. Shared System (PIC and Management); (i) (b); Functionally equivalent to a full-shared system except that the PIC and a designated member of management, often the Director of Flight Operations or any suitably qualified and knowledgeable member of management designated by the operator are jointly responsible for the functions, duties or tasks associated with the operational control of a flight. The responsibility to carry out actual functions, duties or tasks such as flight planning, supporting/briefing the crew or flight monitoring is typically assigned to other non-management personnel (e.g. FOOs and/or FOAs).. Non-shared Systems (General); (ii) (a) - (d); Operational control responsibility is assigned only to the PIC who may or may not be assisted by other support personnel.. Non-shared System (PIC-only); Parent provision and/or (ii) (d); The PIC is solely responsible for completing all tasks (unassisted) related to the operational control of each flight. This does not preclude administrative personnel from providing, collecting or assembling operational documents or data related to each flight on behalf of the PIC as defined in Table 3.1. Such systems employ flight monitoring if required by the Authority or desired by the operator.. Non-shared System (PIC-assisted); (ii) (a) - (c); The PIC is solely responsible for all decisions regarding operational control. However, the PIC may be assisted by others, such as an FOA, or an FOO or member of management that functions as an FOA, who is assigned the responsibility to carry out specific functions, duties or tasks, such as flight planning, support, briefing and in-flight monitoring. Such systems employ flight monitoring if required by the Authority or desired by the operator.. Note: - FOOs can be present in shared or non-shared systems of operational control to support, brief and/or assist the PIC or designated member of management in all competencies of operational control. - FOAs can be present in any system of operational control except purely non-shared (PIC only) systems, but their responsibilities are limited to their area(s) of expertise. - FOAs may be assigned specific flight responsibilities depending on area of expertise or general (non-flight specific) responsibilities in support of other operational control personnel or functions. - The responsibilities of administrative personnel utilized in operational control functions are limited to the provision or collection of operational data.. Table 3.1 categorizes operational control personnel, defines their authority, identifies their responsibilities and illustrates the relationship of such responsibilities to the operation as a whole. Table 3.5 defines the competencies of individuals assigned the responsibility for operational control and/or the responsibility to carry out individual operational control functions, duties or tasks. When operational control functions are outsourced to external service providers, an operator would retain overall responsibility for operational control and would ensure such service providers are subjected to contractual and monitoring processes as specified in ORG 2.2.1. FOO and/or FOA responsibilities for operational control typically begin when assigned a flight during flight preparation and end after flight termination.

 DSP 1.3.6

If an FOO is used in the system of operational control, the Operator should assign responsibility to such personnel for: - Assisting the PIC in flight preparation and providing required information; - Assisting the PIC in preparing the operational and ATS flight plans; - When applicable, signing the operational and ATS flight plans; - Filing the ATS flight plan with the appropriate ATS unit; - Furnishing the PIC, while in flight, with appropriate information necessary for the safe conduct of the flight; - In the event of an emergency, initiating relevant procedures as specified in the OM. (GM) Note: . An operator may choose to assign responsibility for one or more of the specified functions to an FOA, or the PIC may be assigned the responsibility for filing the flight plan in the case of iv) and/or for obtaining the necessary information in the case of v). Note: . Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed FOO responsibilities in operational control system (focus: definition of individual functions/duties/tasks assigned to FOO in specific type of shared/non-shared system of operational control). ☐ Interviewed responsible operational control manager(s). ☐ Examined job description for FOO position (focus: position responsibilities appropriate for assigned functions/duties/tasks in specific type of shared/non-shared system of operational control). ☐ Examined training/qualification records of selected FOO personnel (focus: qualifications appropriate for assigned responsibilities in operational control system). ☐ Observed operational control/flight dispatch operations (focus: assignment of functional responsibilities/duties to FOO personnel). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Aircraft Tracking. The specifications of this provision apply to each FOO qualified in all applicable competencies of operational control, whether licensed or not, who participates in an approved or accepted system of operational control and: - Is delegated authority in accordance with DSP 1.3.4, and/or - Is assigned the responsibility in accordance with DSP 1.3.5 to carry out operational control functions, duties or tasks related to all applicable competencies specified in Table 3.5. - The authority and responsibilities of an FOO are defined in Table 3.1. The specification in item v) may be satisfied by the PIC if such information is available from other sources that can be accessed while in flight.

 DSP 1.3.7

The Operator should have a process to be used in the event of an emergency situation that endangers the safety of the aircraft or persons, including those situations that become known first to the Operator. Such process shall ensure the FOO, FOA or other delegated person: - Initiates emergency procedures, as outlined in the OM, while avoiding taking any action that would conflict with ATC procedures; - Notifies the appropriate authorities, without delay, of the nature of the situation; - Requests assistance, if required; - Conveys, by any available means, safety-related information to the PIC that may be necessary for the safe conduct of the flight, including information related to any necessary amendments to the flight plan. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed procedure for implementation of emergency procedures/actions (focus: definition of operational control positions/persons with assigned responsibility for initiating emergency procedures/notifying authorities/requesting assistance). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: preparedness to implement emergency actions). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specification in item ii) refers to notification to the appropriate authorities without delay and/or within a period(s) specified by each applicable authority. Applicable authorities include those authorities that have jurisdiction over international operations conducted by an operator over the high seas or the territory of a state that is other than the State of the Operator. Processes used for operational control of flights in the event of an emergency would typically be compatible with any operating procedures that have been established by the agencies providing system services for air traffic control. Such compatibility is necessary to avoid conflict and ensure an effective exchange of information between the operator and any of the service agencies. During an operational emergency, the procedures specified in item i) would normally be designed to not conflict with ATC procedures, such as separation standards, controller instructions, minimum flight altitude assignments or any other restrictions imposed by ATC. During an emergency, however, the PIC may exercise emergency authority and take any action necessary in the interest of the safety of the passengers and aircraft.

### 1.7 Operations Manual

DSP 1.7.1

The Operator shall have an Operations Manual (OM) for the use of operational control personnel, which may be issued in separate parts, and which contains or references the policies, procedures and other guidance or information necessary for compliance with applicable regulations, laws, rules and Operator standards. As a minimum, the OM shall: - Be managed and controlled in accordance with ORG 2.5.1; - Have all parts relevant to operational control personnel clearly identified and defined; - Be in accordance with the specifications in Table 3.2. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed operational documents that comprise the OM (focus: external documents referenced in OM/used by operational control personnel). ☐ Interviewed responsible operational control manager(s). ☐ Examined selected parts of OM (focus: contents in accordance with in Table 3.2). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure operational control personnel are able to find all information necessary to perform their functions either within the OM or within another document that is referenced in the OM. The OM is normally identified as a source of operational information approved or accepted for the purpose by the operator or the State. Human factors are considered in the design of the OM to achieve the following: - Preparation of documentation in a useable format for information presentation, at the appropriate reading level and with the required degree of technical sophistication and clarity. - Improving user performance through the use of effective and consistent labels, symbols, colors, terms, acronyms, abbreviations, formats and data fields. - Ensuring the availability and usability of information to the user for specific tasks, when needed, and in a form that is directly usable. - Designing operational procedures for simplicity, consistency and ease of use. - Enabling operators to perceive and understand elements of the current situation and project them to future operational situations. - Minimizing the need for special or unique operator skills, abilities, tools or characteristics. - Assessing the net demands or impacts upon the physical, cognitive and decision-making resources of the operator, using objective and subjective performance measures.

 DSP 1.7.2

The Operator should have a description of the Operational Flight Plan (OFP) or equivalent document that is published in the OM and includes: - Guidance for use by operational control personnel; - An outline of the content in accordance with specifications in Table 3.3. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed description of OFP in OM. ☐ Examined selected OFP(s). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Items readily available in other documentation, obtained from another acceptable source or irrelevant to the type of operation may be omitted from the OFP.

### 1.8 Records System

 DSP 1.8.4

The Operator should have a process or procedures to record and retain operational information, communications and data for each flight. As a minimum, such retained flight information and data shall be in accordance with the specifications in Table 3.4 and retained for a period of time determined by the Operator or the Authority. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed process or procedures for management/control of records in operational control system (focus: retention of information/data for each flight as specified in Table 3.4). ☐ Interviewed responsible operational control manager(s). ☐ Examined selected operational control records (focus: information/data for each flight as specified in Table 3.4.) ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Operational information and data may be retained by different means (e.g. ACARS logs, paper logs, manually, computer systems). Fuel and oil consumption records are typically maintained in accordance with MNT 3.1.1. The communications typically subject to the record keeping specifications of this provision include operational voice, text, or data communications to/from: - Flights from the period beginning at the originating station when flight crew begins their duties on the flight deck until the flight crew finishes their duties on the flight deck at the terminating station; - If applicable, the operations control center. Aircraft tracking data is typically retained only for the purposes of determining an aircraft's position in the event of an accident.

## 2 Training and Qualification

### 2.1 Training and Evaluation Program

#### General

DSP 2.1.1

The Operator shall have a training program, approved or accepted by the Authority, to ensure the operational control personnel specified in Table 3.1, as applicable, are competent to perform any assigned duties relevant to operational control in accordance with the applicable specifications of Table 3.5. Such program shall, as a minimum, address: - Initial qualification; - Continuing qualification. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed training program for operational control personnel (focus: program addresses initial/continuing qualification for functions specified in Table 3.1). ☐ Interviewed responsible operational control manager(s). ☐ Examined training/qualification course curricula for operational control personnel (focus: course content as specified in Table 3.5). ☐ Examined training/qualification records of selected operational control personnel (focus: completion of initial/recurrent training). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Continuing Qualification, State Acceptance and State Approval. Not all states require the approval or acceptance of a training program for operational control personnel. In such cases, state acceptance is considered implicit. A training program for operational control personnel typically addresses: - For FOO and FOA personnel, initial and recurrent training in accordance with the specifications of Table 3.1 and Table 3.5; - For FOO and FOA personnel, a method of qualification through written, oral and/or practical evaluation; - For administrative support personnel as defined in Table 3.1, on-the-job training (OJT), job descriptions, task cards, guidelines, checklists, training materials or other written means to establish competence. The training curriculum normally specifies minimum training hours for each subject area and also indicates whether it has been mandated by the Authority or operator. The specifications of this provision apply to FOO or FOA personnel who are delegated authority and/or assigned responsibilities in accordance with DSP 1.3.4 and/or DSP 1.3.5, respectively. FOO personnel who have completed training programs conducted in accordance with ICAO Doc 7192-AN/857, Part D, Training Manual–Flight Operations Officers/Flight Dispatchers, Second Edition, meet the specifications of this provision. FOO initial training programs contain all of the competencies in Table 3.5 that are relevant to the operations of the operator. FOA initial training programs contain the competencies in Table 3.5 that are relevant to their job function as determined by the operator. Different methods of conducting recurrent training are acceptable, including formal classroom study, home study, computer-based training, seminars and meetings. All recurrent training, regardless of method, is documented and retained in accordance with ORG 2.6.1.

## 3 Line Operations

DSP 3.1.2

The Operator shall have a process or procedures to ensure the PIC is provided with all documents, information and data necessary for the safe conduct of the flight. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed operational control process/procedure for provision of documentation to flight crew (focus: definition of required documents/information/data provided to flight crew). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: PIC provided with documents/information/data necessary for safe conduct of flight). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The OM typically specifies the documents required by the PIC for the safe conduct of each flight. This list of required documents may also be replicated on the folder/envelope containing such documents or displayed in the operational control/flight dispatch center/office for reference purposes. Additionally, the process or procedures associated with the provision of flight documents typically includes safeguards to ensure all of the required documents are provided to the PIC prior to each flight.

### 3.2 Flight Preparation and Planning



 DSP 3.2.5

The Operator should have guidance and procedures that ensure the original OFP or equivalent document is accepted and signed by the following personnel, using either manuscript or an approved electronic method: - The PIC for all systems of operational control; - The FOO for a shared system of operational control; - Designated member of management or post holder in a shared system of operational control that requires the use of such management personnel. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified guidance/procedures for acceptance of OFP (focus: method of signature acceptance for OFP amendments). ☐ Identified/Assessed guidance/procedures for acceptance of OFP (focus: method of signature acceptance for OFP amendments). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: acceptance of OFP by PIC and, if applicable, FOO or designated member of management). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance In a shared system of operational control, the signatures (manuscript or electronic) of both the PIC and the FOO or, if applicable, the designated member of management, are required on the OFP or equivalent document (e.g. dispatch release). The specification in item iii) refers to a designated member of management in a shared system of operational control (e.g. director of flight operations or other designated post holder). Refer to Table 3.1 for the definitions of authorities and responsibilities associated with operational control personnel.

 DSP 3.2.7

If an FOO or FOA is used in the system of operational control, the Operator should have a process or procedures to ensure Operator changes in an ATS flight plan are, when practicable, coordinated with the appropriate ATS unit before transmission to the aircraft by the FOO, FOA or other delegated person. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified use of FOO/FOA in operational control system (focus: applicable to FOO/FOA functions as defined in Table 3.1). ☐ Identified/Assessed process/procedures for coordination of ATC flight plan changes (focus: FOO/FOA coordinates changes with ATC prior to flight plan transmission to flight crew). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: coordination of changes to ATS flight plan by FOO/FOA). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The specifications of this provision apply to FOO or FOA personnel who are delegated authority and/or assigned responsibilities in accordance with DSP 1.3.4 and/or DSP 1.3.5 respectively. The intent of this provision is to ensure ATS flight plan changes that occur prior to departure and/or en route are, when practicable, coordinated with the appropriate ATS unit prior to transmission to the aircraft. When such coordination is not possible, the flight crew remains responsible for obtaining an appropriate clearance from an ATS unit, if applicable, before making a change in flight plan.

 DSP 3.2.9A

If the Operator is authorized to conduct certain portions of a commercial flight under visual flight rules (VFR), the Operator should have guidance and procedures that: - Specify the type of flight plan to be filed with the appropriate ATS unit; - Require current meteorological reports, or a combination of current reports and forecasts, to indicate that meteorological conditions along the portion of the flight to be flown under VFR will, at the appropriate time, be such as to make compliance with VFR possible. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified authorization for portions of flights to be conducted under VFR. ☐ Identified/Assessed guidance/procedures applicable to conducting portions of flights under VFR (focus: flight planning accounts for type of flight plan to be filed/required meteorological conditions; determination of expected times when meteorological conditions will permit compliance with VFR). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: guidance/procedures for control of flights to be conducted under partial VFR, availability of meteorological reports, determination of expected times/conditions that will permit compliance with VFR). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure: - Operations that require compliance with VFR are practicable under the anticipated meteorological conditions; - The operator has guidance and procedures for determining expected times when meteorological conditions will permit compliance with VFR. The specification in item i) refers to the type of flight plan to be filed in instances where certain portions of a flight will be conducted under VFR. In some cases, it may be possible to identify VFR portions in a predominantly instrument flight rules (IFR) flight plan (e.g. Y and Z designation on an ICAO flight plan). Guidance related to the filing of a composite ICAO flight plan, and the use of the Y designation for flights initially operated under IFR and Z designation for flights initially operated under VFR, is contained in Amendment 1 to the Procedures for Air Navigation Services–Air Traffic Management (PANS-ATM, Doc 4444).

 DSP 3.2.9B

The Operator should have guidance and procedures to ensure a flight to be conducted in accordance with IFR does not: - Take off from the departure airport unless the meteorological conditions are at or above the operator's established airport takeoff operating minima for that operation; and - Take off, or continue beyond the point of in-flight re-planning, unless at the airport of intended landing or at each required alternate airport, current meteorological reports or a combination of current reports and forecasts indicate that the meteorological conditions will be, at the estimated time of use (ETU), at or above the operator's established airport operating minima for that operation. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified guidance/procedures for the assessment of airport meteorological conditions prior to departure of IFR flights (focus: flight planning determines that conditions at departure/destination/alternate airports meet all applicable requirements). ☐ Identified/Assessed process/procedures for coordination of ATC flight plan changes (focus: FOO/FOA coordinates changes with ATC prior to flight plan transmission to flight crew). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations office (focus: procedures for monitoring/assessing meteorological conditions for operational airports). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definitions of Alternate Airport, In-flight Re-planning Point and Estimated Time of Use (ETU). The intent of this provision is to ensure: - Flights do not take off or continue beyond the point of in-flight re-planning unless the meteorological conditions at each airport specified in i) or ii), are or will be at or above the operator's established airport operating minima for the operation at the ETU; - The operator has guidance and procedures for determining the ETU. The ETU specified in (ii) is typically the estimated time of arrival derived from the OFP. However, some operators may apply a time margin as required by the State. The specification in item ii) would require the definition and application of alternate airport planning minima in accordance with DSP 3.2.9C.

 DSP 3.2.9C

The Operator should have guidance and procedures, approved or accepted by the State, for determining whether an approach and landing can be safely conducted at each required alternate airport at the ETU. Such guidance and procedures shall specify the appropriate incremental values for visibility (and ceiling, if required), to be added to the Operator's established airport operating minima. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed guidance/procedures for the application of safety margins in the assessment/selection of planned alternate airports (focus: flight planning takes into account defined additives/margins to alternate airport operating minima/times of arrival to account for forecast uncertainties, determination of ETU). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: coordination of changes to ATS flight plan by FOO/FOA). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is for the operator to have a means to ensure, with a reasonable degree of certainty, that at the ETU of an alternate airport, the meteorological conditions will be at or above the operator's established operating minima for an instrument approach. This is practically accomplished through guidance and procedures for the definition and application of alternate planning minima and the determination of an ETU. The specified visibility (and, if required, ceiling) additives are typically dependent on the approach facility configuration and State requirements for a ceiling to be taken into account. The ETU for alternate airports is normally determined in accordance with the type of operational control system and requirements of the State: - In a non-shared system of operational control, the ETU is typically expressed as a time margin (e.g. one hour before to one hour after the ETA at the alternate airport); - In a shared system of operational control, the ETU is typically considered to be a specific point in time coupled with a requirement to ensure the alternate airport remains at or above appropriate minima for the duration of the flight. When determining an ETU, the operator might use a variable time margin based on specific flight parameters that can be monitored after departure by an FOO or FOA and communicated to the PIC. An operator, in accordance with the requirements of the Authority, typically uses technical guidance for the development or application of alternate airport planning minima. Such guidance might be derived from one or more of the following source references, as applicable: - ICAO Flight Planning and Fuel Management Manual (Doc 9976). - Commission Regulation EC No. 965/2012. - FAR 121.625–Alternate Airport Weather Minima. - FAR 121.631(b)–Original Dispatch or Flight Release, Redispatch or Amendment of Dispatch or Flight Release. - FAA OPSPEC C055 Table. - Any equivalent reference document approved or accepted by the Authority for the development or application of alternate planning minima designed to conform to the specifications of the provision.

### 3.3 Aircraft Performance and Load Planning

DSP 3.3.1

The Operator shall have guidance and procedures to ensure a planned flight does not exceed: - The maximum performance takeoff, en route and landing weight limits, based upon environmental conditions expected at the times of departure, along the route of flight and at arrival; - The aircraft structural ramp, takeoff and landing weight limits. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed guidance/procedures for application of aircraft performance data for planned flights (focus: flight planning accounts for aircraft takeoff/en route/landing performance weight limitations). ☐ Interviewed responsible operational control manager(s). ☐ Coordinated with FLT auditor (focus: preflight consideration of aircraft performance limitations). ☐ Observed operational control/flight dispatch operations (focus: guidance/procedures/restrictions that ensure flights do not exceed aircraft performance weight limitations). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure the presence of guidance and procedures for the calculation of maximum takeoff and landing weights, based on takeoff, en route, landing performance, structural limitations as well as any applicable MEL restrictions. Additionally, such guidance and procedures address the means used to prevent an aircraft from being loaded in a manner that precludes a flight from being operated overweight (e.g. notification of weight restrictions to a Load Control Center/office or equivalent).

### 3.4 Icing Conditions

 DSP 3.4.1

The Operator should have guidance and procedures to ensure a flight to be operated in known or expected icing conditions shall not be commenced unless the aircraft is certificated and equipped to be operated in such conditions. Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed guidance/procedures for consideration of aircraft type for flights planned into expected in-flight icing conditions (focus: flight planning accounts for aircraft certified/equipped for icing conditions). ☐ Interviewed responsible operational control manager(s). ☐ Examined selected flight planning records (focus: aircraft certified/equipped for flight into icing conditions). ☐ Coordinated with MNT auditor (focus: verification of fleet(s) certified/equipped for in-flight icing conditions; identification of any exceptions). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flights planned to operate in known icing conductions are only conducted using appropriately certificated and equipped aircraft, which includes consideration of inoperative items on the Minimum Equipment List (MEL). Additionally, if the operator uses a mixed fleet including aircraft that are and are not suitably equipped for operations in icing conditions, the operator would have a means to preclude unequipped aircraft from being used on flights in known icing conditions.

### 3.7 Emergency Response

 DSP 3.7.3

If the Operator transports dangerous goods as cargo, the Operator should ensure FOO, FOA and/or other designated operational control personnel: - Have access to the same information pertaining to dangerous goods carried as cargo on board the aircraft that is provided to the PIC; - Are assigned the responsibility to provide detailed information without delay about dangerous goods carried as cargo to emergency services responding to an accident or serious incident involving the Operator's aircraft. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified authority for transport of dangerous goods as cargo. ☐ Identified/Assessed guidance/procedures for notification to emergency services responding to an aircraft accident (focus: procedures/responsibility for providing timely dangerous goods information). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: access to same dangerous goods information as provided to PIC; preparedness to provide dangerous goods information in event of accident). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure flights planned to operate in known icing conductions are only conducted using appropriately certificated and equipped aircraft, which includes consideration of inoperative items on the Minimum Equipment List (MEL). Additionally, if the operator uses a mixed fleet including aircraft that are and are not suitably equipped for operations in icing conditions, the operator would have a means to preclude unequipped aircraft from being used on flights in known icing conditions.

## 4 Operational Control Requirements and Specification

### 4.1 Alternate and Isolated Airports



DSP 4.1.1

The Operator should have a system, process and/or procedures for alternate airport selection to ensure an appropriate takeoff alternate airport is selected and specified on the OFP whenever: - The meteorological conditions at the airport of departure are below the applicable airport operating landing minima, and/or; - Other operational conditions exist, as defined by the State or the Operator, that would preclude a return to the departure airport. (GM) Note: . The Operator may conform with this provision through Operational Variations approved by the Authority or other variations to takeoff alternate airport selection criteria provided each variation is subjected to the Operator’s SRM processes and safety performance monitoring to ensure an acceptable level of safety is maintained. Note: . Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed system/process/procedures for takeoff alternate airport selection (focus: flight planning includes assessment/selection/designation on OFP of takeoff alternate airport when meteorological/other conditions preclude flight return to departure airport). ☐ Examined selected OFPs (focus: designation of takeoff alternate airport). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: process for selection of takeoff alternate airports). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of Operational Variations. The intent of this provision is to ensure a methodology exists for the selection and specification of takeoff alternate airports when required. The selection of such airports is typically intended to address an operational condition (e.g. an emergency during or immediately after takeoff) that would require the flight crew to land the aircraft as soon as practicable. Accordingly, the applicable operating landing minima specified in the provision would typically refer to the minimum ceiling and/or visibility/runway visual range for landing with an engine inoperative as established by the operator. Takeoff alternates are typically selected during the planning stage but may be selected after flight commencement when necessary via radio, ACARS, or any other communication means acceptable to the operator and the State. An operator may use a system, a process or procedures alone or in any combination in order to fulfill operational requirements related to the selection of takeoff alternate airports. In all cases, however, the robustness of any methodologies used for takeoff alternate airport selection is commensurate with the breadth and complexity of the operation. A description of the typical relevant organizational and operational capabilities related to the use of variations, including Operational Variations approved by the Authority, can be found in the General Guidance at the beginning of this subsection. Variations to alternate airport selection are typically approved or accepted by the State. The subordinate SRM processes of an existing organizational SMS can be applied to variations to ensure the desired level of safety is being achieved. Such SRM processes would typically consider at least the following: - Capabilities of the operator; - Overall capability of the aircraft and its systems; - Available airport technologies, capabilities and infrastructure; - Quality and reliability of meteorological information; - Identified hazards and safety risks associated with each alternate aerodrome variation; and - Specific mitigation measures. Guidance on safety risk management and performance of safety risk assessments is contained in the ICAO Safety Management Manual (SMM) (Doc 9859). Examples of variations related to the selection of alternate airports are contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

 DSP 4.1.5

The Operator should have a system, process and/or procedures for alternate airport selection that takes into account meteorological conditions and relevant operational information to ensure a second destination alternate airport is specified on the OFP and the ATS flight plan under one or more of the following conditions (as approved or accepted by the Authority based on the operations of the Operator): - When, for the destination airport, meteorological conditions at the ETU will be below the Operator's established airport operating minima. - When, for the destination airport, meteorological information is not available (unless the Authority will not permit the initiation of a flight in the absence of such information). (GM) - If the Operator conducts operations to airports with “marginal” meteorological conditions as defined in the OM, when, for such operations, the meteorological conditions at the ETU of the destination and first alternate airports will be marginal. - If the Operator conducts extended over-water operations as defined in the OM, when, for such operations, the meteorological conditions at the ETU of the destination airport will be below the Operator's established operating minima for that operation, unless there is a reasonable certainty that the first alternate airport will be at or above the Operator's established operating minima at the ETU. (GM) Note: . The Operator may conform with this provision through Operational Variations approved by the Authority or other variations to destination alternate airport selection criteria provided each variation is subjected to the Operator’s SRM processes and safety performance monitoring to ensure an acceptable level of safety is maintained Note: . Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed system/process/procedures for selection of a second destination alternate airport (focus: flight planning takes into account regulatory/operational conditions/ requirements/factors applicable to the operator/flight; such conditions/requirements/factors that are considered/assessed in the destination alternate airport selection process are defined). ☐ Examined selected OFPs/ATS flight plans (focus: designation of second destination alternate airport in accordance with relevant factors). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: system/process for selection/designation of second destination alternate airport) ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The principal intent of this provision is to address the safety risks associated with lack of weather reporting for the destination airport or its unavailability at the ETU due to the prevailing meteorological conditions. As a practical matter this may be accomplished by the selection and specification of a second alternate in accordance with the technical specifications of the provision and/or to otherwise ensure, to the extent reasonably practicable, that an airport of intended landing will be available to a flight at the ETU. An operator may conform to a minimum of one of the numbered specifications of the provision and be in overall conformity with the intent of the entire provision. Individual conformity with items i) through iv) is “as applicable to the operator” and dependent on many factors including the regulatory environment and the type of operations conducted. The specifications in ii) define a condition that triggers the selection and specification of a second destination alternate except in cases when the operator is not authorized to depart in the absence of any destination weather information. In such cases, the Authority may authorize departures without nominating a second destination alternate if, for example: - The FOO and flight crew obtain and consider those weather reports and forecasts which are available; - The FOO and flight crew ensure adequate contingency plans (such as extra fuel) are available to deal with an unfavorable change in conditions. The term “marginal” as used in item (iii) is typically not defined by regulation. This, to some extent, is because the definition of what constitutes “marginal” depends on the nature of the meteorological conditions present, the type of operation being conducted and the capabilities of the airborne and ground-based equipment available. In any case, an operator, in order to conform to item iii) must clearly define the term including the conditions under which a second alternate is required. The specifications in item iii) are typically applicable to flights conducted between airports within the territories of one nation or country, or between nearby countries as approved or accepted by the applicable authorities. The specification in item iv) is applicable if the term “extended overwater operations” is defined by regulation of the State and by the operator. Such term is typically defined as an operation over water at a horizontal distance of more than 50 nautical miles from the nearest shoreline. An operator may use a system, process and/or procedures alone or in any combination in order to fulfill operational requirements related to the selection of alternate airports. In all cases, however, the robustness of any methodologies for destination alternate airport selection is commensurate with the breadth and complexity of the operation. A description of the typical relevant organizational and operational capabilities related to the use of variations, including Operational Variations approved by the Authority, can be found in the General Guidance at the beginning of this subsection. Variations to alternate airport selection are typically approved or accepted by the State. The subordinate SRM processes of an existing organizational SMS can be applied to variations to ensure the desired level of safety is being achieved. Such SRM processes would typically consider at least the following: - Capabilities of the operator; - Overall capability of the aircraft and its systems; - Available airport technologies, capabilities and infrastructure; - Quality and reliability of meteorological information; - Identified hazards and safety risks associated with each alternate aerodrome variation; and - Specific mitigation measures. Guidance on safety risk management and performance of safety risk assessments is contained in the ICAO Safety Management Manual (SMM) (Doc 9859). Examples of variations related to the selection of alternate airports are contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

### 4.2 Minimum Flight Altitudes and En Route Performance



DSP 4.2.1

The Operator should have guidance and procedures to ensure planned minimum flight altitudes are not less than those established by the applicable authorities. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed guidance/procedures for planning altitudes for all flights (focus: flight planning takes into account and ensures flights meet minimum altitude limitations established by regulation). ☐ Examined selected OFPs/flight records (focus: planned flight altitudes within minimum altitude limits). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: flight planning; altitude selection). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Operational flight planning includes a review of the route of flight, in conjunction with published aeronautical information, to ensure compliance with minimum flight altitudes. Such review could include: - Minimum Safety Altitude (MSA); - Minimum Descent Altitude/Height (MDA/H); - Minimum En route Altitude (MEA); - Minimum Obstruction Clearance Altitude (MOCA); - Minimum Off-Route Altitude (MORA); - Minimum Vectoring Altitude (MVA); - Any other minimum altitudes prescribed by the Authority. Applicable authorities include those authorities that have jurisdiction over international operations conducted by an operator over the high seas or the territory of a state that is other than the State of the Operator.

### 4.3 Fuel Planning

DSP 4.3.1

The Operator shall have a system, process and/or procedures to ensure an aircraft carries a sufficient amount of usable fuel to complete each planned flight safely and allow for deviations from the planned operation. (GM)

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Finding) ☐ Implemented not Documented (Finding) ☐ Not Documented not Implemented (Finding) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed system/process/procedures for fuel planning for all flights (focus: flight planning takes into account possible deviations from planned operation in calculating usable fuel for safe completion of flight). ☐ Interviewed responsible operational control manager(s). ☐ Examined selected OFPs (focus: fuel load meets/exceeds minimum required departure/dispatch fuel). ☐ Observed operational control/flight dispatch operations (focus: process or procedures that ensure sufficient usable fuel for safe flight completion taking into account unplanned deviations). ☐ Coordinated with flight operations (focus: complementary procedures for assessing minimum required fuel). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to define the foundation necessary to support the practical implementation of an operator's fuel policy. It also addresses the baseline criteria to be considered in any methodology used in the determination of total usable fuel required to complete each planned flight safely. Simply put, it requires an operator to use system, process and/or procedures alone or in any combination in order to fulfill operational requirements related to the implementation of its fuel policy. In all cases the robustness of any such methodologies is commensurate with the breadth and complexity of the operation and takes into account: - The aircraft-specific data and operating conditions for the planned operation. - The following components of usable fuel required in accordance with the respective provisions of this sub-section: - Taxi fuel; - Trip fuel in; - Contingency fuel; - If required (as applicable to each flight): - Destination alternate fuel, or - No-alternate fuel or - Isolated airport fuel - Final reserve fuel; - If required, additional fuel; - If requested by the PIC, or the PIC and FOO in a shared system of operational control, discretionary fuel. Some regulatory authorities or operators may classify destination alternate fuel, no alternate fuel and Isolated airport fuel under the common heading of “Alternate Fuel” in regulations and/or flight planning systems. It is important for operational control personnel and the flight crew to have a clear and common understanding of the terms used in the operator's fuel policy, as such understanding is the key to successful flight planning and completion. Equally important is the notion that differences in terminology may exist from operator to operator. Regardless of the terms used, however, an operator can conform to the provisions of this sub-section if the pre-flight computation of usable fuel is substantially equivalent, allocates fuel in a similar fashion, and has the components that, when combined, result in an equivalent or greater amount of fuel. Fuel calculations are typically made by a flight crew member, a Flight Operations Officer/Flight Dispatcher (FOO), or both. Guidance on the organizational and operational systems and processes related to the implementation of fuel policy is contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

 DSP 4.3.2

The Operator should have a system, process and/or procedures to ensure the amount of usable fuel to be carried on an aircraft in accordance with DSP 4.3.1 is, as a minimum, based on the following data and operating conditions for each planned flight: - Current aircraft-specific data derived from a fuel consumption monitoring program, if available, or if current aircraft-specific data is not available, data provided by the aircraft manufacturer; - The anticipated aircraft mass; - Notices to Airmen (NOTAM); - Current meteorological reports, or a combination of current reports and forecasts; - Applicable air traffic services procedures, restrictions and anticipated delays; - The effects of deferred maintenance items and/or configuration deviations; - Any other conditions that might cause increased fuel consumption. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed system/process/procedures for planning sufficient usable fuel for safe completion of all flights (focus: flight planning takes into account operating data/conditions that might cause/lead to increased fuel consumption; such operating data/conditions that are considered/assessed in usable fuel calculation process are defined). ☐ Examined fuel policy (focus: guidance for calculation of minimum required departure/dispatch fuel). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: process or procedures that ensure planned flight usable fuel is based on all relevant data/operating conditions). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance Refer to the IRM for the definition of NOTAM (Notice to Airmen). The intent of this provision is to define the aircraft-specific data, manufacturer data, operating conditions and other factors that would be considered by an Operator during the pre-flight computation of the total usable fuel required for a planned flight. When considered in combination with DSP 4.3.1, this provision helps to form the basic foundation for the means to complete the preflight calculation of usable fuel. The specification in item i) refers to the process for ensuring actual aircraft fuel use approximates planned fuel use within an acceptable margin of error. This is practically accomplished by comparing the achieved in-flight performance of an aircraft to its predicted performance. Variations between the achieved performance and the predicted performance will result in a variation of the rate of fuel consumption which is typically accounted for by the operator during flight planning and in flight. An operator may use a system, process and/or procedures alone or in any combination in order to fulfill operational requirements related to the implementation of fuel policy. In all cases, however, the robustness of any such methodologies is commensurate with the breadth and complexity of the operation. Guidance on fuel planning including guidance related to the creation and maintenance of fuel consumption monitoring programs is contained in the ICAO Flight Planning and Fuel Management Manual (Doc 9976).

 DSP 4.3.12

The Operator should have a process and/or procedures to ensure the final reserve fuel calculated in accordance with its fuel policy is not less than the amount of fuel required to fly for 30 minutes under speed and altitude conditions specified by the Operator and as approved or accepted by the Authority. (GM) Note: Effective 1 April 2025, this recommended practice will be upgraded to a standard.

☐ Documented and Implemented (Conformity) ☐ Documented not Implemented (Observation) ☐ Implemented not Documented (Observation) ☐ Not Documented not Implemented (Observation) ☐ N/A

Auditor Comments Click or tap here to enter text.

Auditor Actions ☐ Identified/Assessed process/procedures for calculation of final reserve fuel for all flights (focus: planned final reserve fuel is an amount that is not less than fuel to fly for 30 minutes at holding speed at 450 m/1500 ft or fuel to fly 30 minutes under speed/altitude conditions approved/accepted by authority). ☐ Examined selected OFPs (focus: factors used as basis for final reserve fuel). ☐ Interviewed responsible operational control manager(s). ☐ Observed operational control/flight dispatch operations (focus: process or procedures for calculating planned final reserve fuel). ☐ Other Actions (Specify) Click or tap here to enter text.

Guidance The intent of this provision is to ensure the allocation of an amount of fuel to be protected in flight and preserved upon landing at any airport. As such, it represents the last line of defense in a multilayered strategy to ensure safe flight completion. An operator may define the 30-minute final fuel reserve requirements using speed, altitude and/or other conditions that are in accordance with requirements of the Authority (e.g. 30 minutes at holding speed at 450m/1,500 ft above airport elevation in standard conditions).

| Table 3.1–Operational Control Personnel |  |  |  |
| --- | --- | --- | --- |
| This table categorizes operational control personnel, defines the scope of their authority, identifies their responsibilities and illustrates the relationship of such responsibilities to the operation as a whole. It shall be used for the purposes of applying relevant Section 3 provisions and is provided to ensure suitably qualified persons are designated, where applicable, to support, brief and/or assist the pilot-in-command (PIC) or FOO or designated member of management in the safe conduct of each flight. The terms used in the table to identify operational control personnel are generic and might vary. Personnel, however, employed in operational control functions that are delegated the authority and/or assigned the responsibility to carry out functions, duties or tasks, as outlined in the table, are subject to the training and qualification requirements commensurate with their position. |  |  |  |
| Operational Control | Authority (DSP 1.3.4) | Responsibilities, Including the Assignment of Functions, Duties or Tasks (DSP 1.3.5) | Training and Qualification Operator shall designate responsibilities and ensure personnel are competent to perform the job function. |
| Administrative Support Personnel1 (e.g. gate agent) | None Do not make recommendations or decisions regarding the operational control of a flight. | Provide, collect or assemble operational documents or data only. | Not subject to initial and recurrent training in the competencies of operational control in Table 3.5 and are qualified via On the Job Training (OJT), job descriptions, task cards, guidelines, checklists, training materials or other written means to establish competence. |
| Flight Operations Assistant (FOA)4 (e.g. Weather Analysts, Navigation Analysts/Flight Planning Specialists, Load Agents/Planners, Operations Coordinators/Planners, Maintenance controllers, Air Traffic Specialists, Planners/Controllers unless qualified in accordance with GRH) | None or limited to area(s) of expertise May be authorized to make decisions or recommendations in area(s) of expertise.5 ( e.g., maintenance controller grounds aircraft.) | Support, brief and/or assist the PIC or FOO. Specializes in one or more of the elements of operational control.3 Collects, provides filters, evaluates and applies operational documents or data relevant to specific elements of operational control. Makes recommendations or decisions in area(s) of expertise. | For each area of expertise or specialization3 Subject to initial and recurrent training in accordance with DSP 2.1.1specific competencies of Table 3.5 relevant to the job function and operations of the Operator. |
| Flight Dispatcher or Flight Operations Officer (FO O)4 or Designated Member of Management (e.g. Director of Operations or other nominated Post Holder) | None or limited or shared2 May share operational control authority with the PIC.2 May be authorized to make recommendations or decisions.5 | May share operational control responsibility with the PIC.2 Support, brief, and/or assist the PIC. Collects, provides, filters, evaluates and applies operational documents or data relevant to all elements of operational control.3 Makes recommendations or decisions. | Subject to initial and recurrent training in accordance with DSP 2.1.1 and all competencies of Table 3.5 relevant to the operations of the Operator. |
| Pilot in Command (PIC) | Full/shared2 Has final authority to ensure the safe operation of the aircraft. May share authority and responsibility for operational control. | Full/shared2 Responsible for safe conduct of the flight. Collect, provide, filter, evaluate and applies operational documents or data relevant to all competencies of operational control.3 | Subject to training and qualification requirements specified in ISM Section 2. |
| Legend | 1 - Personnel lacking any authority or responsibility for operational control are identified in the table for the purposes of excluding them from the training and qualification provisions of this section. |  |  |
|  | 2 - FOO personnel used in conjunction with a shared system of operational share authority and responsibility with the PIC. |  |  |
|  | 3 - The competencies of operational control are contained in Table 3.5. FOA personnel that specialize in one competency of operation control may be referred to as Weather Analysts, Navigation Analysts/Flight Planners, Operations Coordinators/Planners, Maintenance controllers, Air Traffic Specialists and Load Agents/Planners/Controllers unless qualified in accordance with GRH. |  |  |
|  | 4 - The terms used in this table to identify operational personnel are generic and may vary. Personnel utilized in operational control functions and assigned the responsibilities delineated in the table are subject to the relevant qualification and training provisions in this section. |  |  |
|  | 5 - Authority limited in scope to decision making in area of expertise. |  |  |

| Table 3.2–Operations Manual (OM) Content Specifications |  |  |  |
| --- | --- | --- | --- |
| This table contains the fundamental OM content specifications required to achieve conformity with DSP 1.7.1. It also specifies Section 2 (FLT) provisions that must be addressed in the sections of the OM relevant to personnel with responsibilities related to the operational control of flights. Note: Specific policies, guidance, data and/or procedures that must be addressed in the sections of the OM relevant to operational control personnel can be found in individual Section 3 provisions and are not duplicated in the table. |  |  |  |
| General Information |  |  | FLT ISARP |
| (i) | General Operations Manual (GOM), to include: |  | None |
|  | (a) | Non-aircraft type related and/or standard operating procedures for each phase of flight, policies, procedures, checklists, descriptions, guidelines, emergency procedures and other relevant information; | None |
|  | (b) | Authorities, duties and responsibilities associated with the operational control of flights; | None |
|  | (c) | If applicable, the requirement for commercial and non-commercial flights to be conducted under an IFR flight plan and in accordance with an IFR flight plan. | FLT 3.10.1 |
| Aircraft Operating Information |  |  | FLT ISARP |
| (ii) | Aircraft Operating Manual (AOM), to include: |  | None |
|  | (a) | Normal, abnormal/non-normal and emergency procedures, instructions and checklists; | None |
|  | (b) | Aircraft systems descriptions, limitations and performance data. | None |
| (iii) | MEL and CDL, to include applicability and a description of the relationship between the Minimum Equipment List (MEL) and the Master Minimum Equipment List (MMEL); |  | None |
| (iv) | Aircraft specific weight and balance instructions/data; |  | None |
| (v) | Instructions for the conduct and control of ground de/anti-icing operations. |  | FLT 3.9.6 |
| Areas, Routes and Airport Information |  |  |  |
| (vi) | Route and airport instructions and information (departure, destination, en route and destination alternates, to include: |  |  |
|  | (a) | Airway manuals and charts, including information regarding communication facilities and navigation aids; |  |
|  | (b) | Airport charts, including the method for determining airport operating minima, operating minima values for destination and alternate airports and the increase of airport operating minima in case of degradation of approach or airport facilities; |  |
|  | (c) | Airport and runway analysis manual or documents: |  |
|  | (d) | If applicable, flight following requirements and instructions to ensure the PIC notifies the operator of en route flight movement or deviations from the OFP including procedures for loss of communication between the aircraft and the FOO; |  |
|  | (e) | Instructions for the conduct of precision and non-precision approaches, including approach minima; |  |
|  | (f) | If applicable, procedures for the conduct of long-range navigation; |  |
|  | (g) | Supplemental oxygen requirements and escape routes in case of decompression in an area of high terrain, if applicable; |  |
|  | (h) | Regional guidance necessary to comply with local regulations. |  |
| Training Information |  |  |  |
| (vii) | Training Manual, to include: |  |  |
|  | (a) | Details of all relevant training programs, policies, directives and requirements, including curricula and syllabi, as applicable, for initial qualification, continuing qualification and other specialized training; |  |
|  | (b) | Curricula for ground training, evaluation and certification; |  |
|  | (c) | Comprehensive syllabi to include lesson plans, procedures for training and conduct of evaluations; |  |
|  | (d) | The training program for the development of knowledge and skills related to human performance (Crew Resource Management/Dispatch Resource Management, CRM/DRM). |  |
| Other Information |  |  |  |
| (viii) | Cabin safety and emergency procedures relevant to operational control personnel. |  |  |
| (ix) | Dangerous Goods manual or parts relevant to operational control personnel, to include information and instructions on the carriage of dangerous goods and action to be taken in the event of an emergency. |  |  |
| (x) | Security Manual or parts relevant to operational control personnel, including bomb search procedures. |  |  |

| Table 3.3–Operational Flight Plan (OFP) Specifications |  |  |
| --- | --- | --- |
|  | The OM contains a description and specifications for the content and use of the OFP or equivalent document. The content of the OFP shall consist of, as a minimum, the following elements: |  |
|  | (i) | Aircraft registration; |
|  | (ii) | Aircraft type and variant; |
|  | (iii) | Date of flight and flight identification; |
|  | (iv) | Departure airport, STD, STA, destination airport; |
|  | (v) | Route and route segments with check points/waypoints, distances and time; |
|  | (vi) | Assigned oceanic track and associated information, as applicable; |
|  | (vii) | Types of operation (e.g. ETOPS/EDTO, IFR, ferry-flight); |
|  | (viii) | Planned cruising speed and flight times between waypoints/check points; |
|  | (ix) | Planned altitude and flight levels; |
|  | (x) | Fuel calculations; |
|  | (xi) | Fuel on board when starting engines; |
|  | (xii) | Alternate(s) for destination and, when applicable, takeoff and en route; |
|  | (xiii) | Relevant meteorological information. |

| Table 3.4–Flight Information |  |  |
| --- | --- | --- |
| The Operator shall record and retain the following information for each flight: |  |  |
|  | (i) | Aircraft registration; |
|  | (ii) | Date; |
|  | (iii) | Flight number; |
|  | (iv) | Flight crew names and duty assignment; |
|  | (v) | Fuel on board at departure, en route and arrival; |
|  | (vi) | Departure and arrival point; |
|  | (vii) | Actual time of departure; |
|  | (viii) | Actual time of arrival; |
|  | (ix) | Flight time; |
|  | (x) | Incidents and observations, if any; |
|  | (xi) | Flight weather briefings; |
|  | (xii) | Dispatch or flight releases; |
|  | (xiii) | Load Sheet; |
|  | (xiv) | NOTOC; |
|  | (xv) | OFP; |
|  | (xvi) | ATS flight plan; |
|  | (xvii) | Communications records; |
|  | (xviii) | Fuel and oil records (obtained in accordance with MNT 3.1.1; |
|  | (xix) | Aircraft tracking data to assist SAR in determining the last known position of the aircraft. |
| Note: After an aircraft has landed safely, an operator may discard tracking data. |  |  |

| Table 3.5–Competencies of Operational Control |  |  |
| --- | --- | --- |
| The Operator shall ensure FOO or FOA personnel demonstrate knowledge and/or proficiency in the competencies of operational control appropriate to the assignment of responsibility to carry out operational control functions, duties, or tasks, to include, as applicable: |  |  |
| Competency |  | FOA Relevancy Examples |
| (i) | Air law | - Air Traffic Specialists |
| (ii) | Flight performance | - As relevant to function |
| (iii) | Navigation | - Navigation Analysts - Flight Planning Specialists |
| (iv) | Aircraft General knowledge and instrumentation | - As relevant to function |
| (v) | Meteorology | - Weather Analysts - Meteorologists |
| (vi) | Mass and balance | - Load Agents - Load Planners - Load Controllers |
| (vii) | Operational procedures | - As relevant to function |
| (viii) | Flight planning and monitoring | - Flight Planning Specialists - Flight Followers |
| Notes |  |  |
| - FOO personnel that are assigned overall operational control responsibility for specific flights, assigned responsibilities in all competencies of operational control or used in shared systems of operational control demonstrate knowledge and/or proficiency in all applicable competencies in this table. - FOO or FOA personnel assigned the individual responsibility to carry out specific operational control functions, duties or tasks demonstrate knowledge and/or proficiency in competencies relevant to area of expertise or function as determined by the operator or State. - It is important to note that some operators might choose to assign the responsibility for specific operational control functions to fully qualified FOO personnel. In such cases an FOO is acting in a limited capacity and although qualified in all competencies of operational control, would be functionally acting as an FOA. |  |  |

| Table 3.6–Guidance for Development of Operational Control Competency Course Syllabi |  |  |
| --- | --- | --- |
| The Operator typically develops a competency course curriculum and related syllabi for each competency in Table 3.5. Curriculum and associated syllabi development can be based on one or more source references or their equivalent: - ICAO Doc 10106 - ICAO Doc 7192 - 14 CFR § 121.415 and 14 CFR § 121.422 - EASA ORO.GEN.110 and related AMC and GM |  |  |
|  | Competency Course Subjects | Examples of Syllabus Outlines (ICAO Doc 10106) |
| (i) | Air law | To enable operational control personnel to identify the basic requirements for authorization to operate a commercial air transportation service, air law may include topics such as: - Conventions and agreements - National organizations and rulemaking process - Air services and airspace - ATC separation and clearances - Search and rescue (SAR) - Security - ATS flight plan (FPL) - Flight safety, accident and incident |
| (ii) | Flight performance | To enable the operational control personnel to identify the basic elements of aircraft performance, flight performance may include topics such as: - Certification standards - Influencing variables on performance - Takeoff performance - Accelerate-stop distance - Balanced field length - Takeoff climb - Obstacle limits - Reduced/de-rated thrust - Cruise - Cost index - Driftdown - Landing performance - Quick turnaround limits |
| (iii) | Navigation | To enable the operational control personnel to identify the fundamentals of navigation and equipment utilized in navigation, navigation may include topics such as: - Basics of general navigation - Latitude, longitude - Time and time conversions - Determining sunrise, sunset, civil twilight - Directions - Distance - Charts - Basics of radio navigation - NDB - VOR - DME - ILS - Radar - GPS/GNSS - RNAV - FMS - RNP - Satellite augmentation systems |
| (iv) | Aircraft General knowledge and instrumentation | To enable the operational control personnel to identify the main components and systems of an aircraft and their basic functions, aircraft general knowledge and instrumentation may include topics such as: - Units and basic definitions - Lift - Drag - Thrust - Weight - Flight mechanics - System design, loads, stresses, maintenance - Hydraulics - Landing gear - Primary and secondary flight controls - Pneumatics - Air conditioning systems - Ice and rain protection - Fuel - Electrics - Engines and APU - Flight management and navigation - Automatic flight - Communications - Fire protection - Equipment and furnishings - Indicating and recording systems |
| (v) | Meteorology | To enable the operational control personnel to interpret meteorological information, reports, forecasts and warnings correctly and efficiently, meteorology may include topics such as: - Atmosphere, composition, extent, vertical division - Air temperature, definition and units - Atmospheric pressure and density - International standard atmosphere (ISA) - Altimetry - Wind - Clouds and fog - Precipitation - Air masses and fronts - Pressure systems - Climatology - Icing conditions - Turbulence - Wind shear - Thunderstorms - Flight hazards - Meteorological information |
| (vi) | Mass and balance | To enable the operational control personnel to identify the basic requirements for load planning, calculation of payload, loadsheet preparation, and aircraft balance, mass and balance may include topics such as: - Importance of structural limitations - Mass terms - Mass limits, structural limitations - Cargo compartment limitations - Mass calculations - Definition of center of gravity (CG) - Load and trim sheet, general considerations |
| (vii) | Operational procedures | To enable the operational control personnel to policies, procedures, guidance, and instructions developed to perform their respective functions, operating procedures may include topics such as: - Operational control responsibilities - SMS - Operating manuals - Aircraft airworthiness - Operational limitations and minima - Duty time limitations and rest requirements - Operational flight plan contents - Anti-icing, de-icing - Security (unlawful events) - Abnormal and emergency procedures - Communication systems and procedures |
| (viii) | Flight planning and monitoring | To enable the operational control personnel to complete an operational flight plan in accordance with laid-down rules and standards and to apply the skills acquired to effectively maintain a flight watch, and monitor fuel consumption, en route weather including winds, aircraft performance including the limitations imposed by MEL restrictions, in-flight equipment failures, security problems, and the effects of and on hazardous materials, restricted articles, and perishable cargo, flight planning and monitoring may include topics such as: - Weather analysis - AIP/NOTAM analysis - Track selection & flight level - Equipment requirements - Airport suitability - Fuel requirements - Payload planning - ETOPS/EDTO - MEL/CDL - ATC/ATM - Security (unlawful events) - Abnormal and emergency procedures |